Master Component Registry for the MAP80-1012  (3-30-2026) JWS

Terminal Blocks, Connectors, Fuses

| Designator | Part Number | Role |
| --- | --- | --- |
| TB1 | 3 Screw Terminal | Primary Line AC Input (Pin-1 Earth Ground) (Pin-2 Neutral) (Pin-3 Line power 120) Note: This Power supply can also have 220-240 VAC between Pin-2 and Pin-3 |
| TB2 | 7 Screw Terminal | Secondary Output: Pin-6 and 7 Secondary Positive (Adjustable 12 to 15 VDC) Pin-4 and 5 Secondary Return/Ground (Pins 1, 2, and 3 are not used on MAP80-1012) |
| J1 | 2 pin JST | Power Fail Output (Open Collector) used only by some equipment as Power OK |
| F1 | 3 AMP Slow Blow | Primary AC Fuse (Soldered in 5x20) |

Semiconductors (Active Components)

| Designator | Part Number | Role | Package |
| --- | --- | --- | --- |
| Q1 | 2SK1081 | Main Switching MOSFET | TO-3P / TO-247 |
| Q2 | PN2222A | Logic Switching (NPN) | TO-92 |
| Q4 | MPSA14 | NPN Darlington Driver | TO-92 |
| Q5 | PN2907A | PNP Control Switch | TO-92 |
| Q6 | PN2222A | Logic Switching (NPN) | TO-92 |
| SCR1 | IS08S | Crowbar Latch Protection | TO-220 |
| U1 | TL431 | Main Precision Reference | TO-92 |
| U5 | TL431 | Protection/UVLO Reference | TO-92 |
| U6 | MOC8104 | Optoisolator Feedback | DIP-4/6 |

Resistors (Passive Network)

| Designator | Value | Role | Note |
| --- | --- | --- | --- |
| V1 | 10 K | Secondary Voltage Adjust | 10 Turn Trim POT |
| RT1 | 10 Ω | NTC Current Inrush Limiter | Vishay SG27 |
| R1 | 1 M | Startup / HV Bleed |  |
| R2 | 5.1K | Logic Bias |  |
| R3 | 6.8 K | Logic Bias |  |
| R4 | 680 Ω | Gate/Base Drive |  |
| R5 | 470 Ω | Drive Limiter |  |
| R6 | 0.12 Ω | Main Current Sense | 3W Metal Strip |
| R7 | 270 Ω | Snubber Dissipation | 5W Cement |
| R8 | 820 Ω | Pull-up/Bias |  |
| R9 | 47 K | Logic Divider |  |
| R10 | 20 K | Control Loop Compensation |  |
| R13 | 47 K | Voltage Feedback Sense |  |
| R16 | 20 Ω | Snubber/Damping |  |
| R17 | 330 Ω | Logic Bias |  |
| R18 | 10 Ω | Damping |  |
| R21 | 500 Ω | Gate Negative Clamp |  |
| R23 | 1.2 K | Sync Pulse Limiter |  |
| R25 | 20 K | Busy Node Timing |  |
| R26 | 200 Ω | Drive Coupling |  |
| R27 | 2.4 K | Opto LED Dropping Resistor |  |
| R28 | 1 M | Safety Bleeder |  |
| R29 | 25 K | Reference Divider |  |
| R30 | 1K | Opto LED Bypass |  |
| R33 | 10 K | Q6 Base drive tied to Secodary 15vdc | U6 collector pulls it down for off |
| R34 | 1K | Logic Rail Feed |  |
| R37 | 150K | Protection Divider |  |
| R38 | 75 Ω | EMI Snubber | On the Inrush limiter node |
| R39 (No Silkscreen) | 270 Ω | Minimum Load Resistor | Mounted in a Hijacked location. Using C18 silkscreen footprint |

Capacitors (Storage & Filtering)

| Designator | Value | Role | Type / Note |
| --- | --- | --- | --- |
| C1 | 0.47 μF | EMI Filtering | X2 Safety 250V |
| C2 | 0.1 μF | EMI Filtering/Snubber | X2 Safety 250V |
| C3 | 4700 pF | EMI Filtering | Y2 Safety 250V |
| C4 | 4700 pF | EMI Filtering | Y2 Safety 250V |
| C5 | 180 µF | Bulk Storage (400V) | Main HV Rail |
| C6 | 180 µF | Bulk Storage (400V) | Main HV Rail |
| C7 | 4700 pF | Noise Filtering (100V) | Ceramic (472k) |
| C8 | 47 µF | Bias / VCC Smoothing (50V) | Electrolytic |
| C10 | 2200 pF | RCD Snubber (3kV) | Ceramic (Z5U) |
| C11 | 47nF | (1KV) |  |
| C12 | 2200 µF | Output Pre-filter (16V) | Secondary |
| C13 | 2200 µF | Output Pre-filter (16V) | Secondary |
| C14 | 2200 µF | Output Pre-filter (16V) | Secondary |
| C16 | 2200 µF | Output Post-filter (16V) | Secondary |
| C21 | 0.01 µF | Noise supression (1kV) | Ceramic (103M) |
| C24 | 1nF | Noise supression (1kV) | Y Safety 250V |
| C26 | 1 nF | Q2 base timing (1kV) | Ceramic (102K) |
| C27 | 47 µF | Aux Supply Reservoir (50V) | Aux (+) to Pri (-) |
| C28 | 3.3 µF | Startup RC Timing | w/ U5 |
| C29 | 22 nF | Snubber / Coupling | T1 Pin 5 to Pin 2 |

Diodes (Steering & Protection)

| Designator | Part Number | Role | Type |
| --- | --- | --- | --- |
| CR1 | KBL06 | Full Wave Bridge | Rectifier |
| CR2 | BYM26E | Energy Scavenging | Fast Rectifier |
| CR3 | 1N5819 | Logic/Aux Supply | Schottky |
| CR4 | 1N961B | Control Rail Reference | 12V Zener |
| CR5 | BYV26C | Snubber Clamping | Ultra-Fast |
| CR7 | BYW29E | Main Secondary Output | Ultra-Fast |
| CR11 | EGP10C | Gate Negative Clamp | Extra-Fast |
| CR12 | 1N5248B | Crowbar Trigger | 18V Zener |
| CR14 | 1N4448 | Logic Steering | Signal Diode |
| CR15 | 1N4448 | Q4 Base Negative Clamp | Signal Diode |
| CR18 | 1N4448 | Steering diode for T2 Red Winding | Signal Diode |
| CR19 | 1N4448 | Creates the VCC node from T1 Pin 3 | Signal Diode |

Magnetics (Transformers and Inductors)

L1 Common Mode Choke (AC Line input power flows through this before getting to the Full Wave Bridge Rectifier CR1)

L2  Inductor (14 Gauge 8 turn Iron core Inductor)

Transformer T1 (Main Power)

- Core: Main Power Ferrite (Large).

- Pins 1 & 2: Primary (Bifilar) Main DC Input / Q1 Drain.

- Pins 3, 5, 6: Primary Aux / Recovery / Ground Center-Tap.

- Pins 11 & 13: Secondary Aux / Logic Sync.

- T1-A & T1-B: Main 15V Output (Flying Leads).

Transformer T2 (Gate Drive / Signal)

Physical Specs: 10mm OD / 4mm ID / 3.07mm Wide (Epoxy Filled).

| Winding | Details | Specs | Function |
| --- | --- | --- | --- |
| Red | 20 Turns | 0.22mm (31 AWG) Magnet Wire | Mosfet Current Feedback |
| White | 2 Turns | 0.4mm (26 AWG) Insulated | Regulation PWM Command (from Q4) |
| Yellow | 2 Turns | 0.4mm (26 AWG) Insulated | Q1 Off signal Galvanically, T2 reset magnetically |